Mini_setup_CREATE / 2_SWATdoctR

SWATdoctR Verification Workflow Manual

# Scope

This document explains only the 2_SWATdoctR part of the Mini_setup_CREATE repository. It is written as a practical manual for teaching a workshop and for documenting how the provided SWAT+ clean setup is checked with SWATdoctR. It does not document the earlier model-building workflow in 1_Setup, except where needed to explain the origin of clean_setup.

Important distinction The 2_SWATdoctR folder does not build the SWAT+ model. It verifies an already prepared SWAT+ TxtInOut-style setup stored in clean_setup. The main task is quality assurance: run diagnostic simulations, extract selected outputs, and generate plots that reveal problems in climate inputs, water balance, management operations, plant growth, plant stress, point sources, or tile drainage.

# Repository folder overview

The 2_SWATdoctR folder is compact. It contains an RStudio project file, one workflow script, and a complete clean_setup folder containing SWAT+ input files. The repository search results show many standard SWAT+ input files inside clean_setup, including climate files, operation files, HRU files, routing/connectivity files, basin files, plant/soil files, reservoir files, and decision-table files.

| Path | Meaning |
| --- | --- |
| 2_SWATdoctR/DoctR.Rproj | RStudio project file. It sets UTF-8 encoding, two spaces for tabs, and normal RStudio project behavior. |
| 2_SWATdoctR/workflow.R | Main SWATdoctR script. It runs three verification simulations and generates plots.pdf. |
| 2_SWATdoctR/clean_setup/ | The SWAT+ model input folder used by run_swat_verification(). It must contain all required model input files and the SWAT+ executable expected by the package/workflow. |
| 2_SWATdoctR/plots.pdf | Expected output generated by workflow.R after all diagnostic plotting commands complete. This file is not an input; it is created by the script. |

# Purpose of the SWATdoctR workflow

SWATdoctR is used here as a model setup verification tool. The goal is not calibration. The goal is to check whether the existing SWAT+ setup behaves plausibly before later calibration, validation, or scenario simulations. The official SWATdoctR workflow describes five major verification areas: climate/input interpretation, management operations, unconstrained plant growth, plant stress effects, and point source or tile drainage outputs.

| Verification area | Main question answered by the workflow |
| --- | --- |
| Climate and weather inputs | Are precipitation, temperature, snow, and annual climate patterns interpreted plausibly by the model? |
| Water balance | Do the water-balance components look physically plausible after the model run? |
| Management operations | Were scheduled agricultural operations actually triggered as expected? |
| Plant growth without stress | Can crops reach plausible maturity, biomass, and yield when stresses are removed? |
| Plant stress with stress modes active | Are low yields caused by water, aeration, temperature, nitrogen, or phosphorus limitations? |
| Point sources and tile drainage | If implemented, are point inputs and tile-flow outputs plausible? |

# Required software and packages

| Requirement | Why it is needed |
| --- | --- |
| R and RStudio | R runs workflow.R; RStudio project file supports workshop execution. |
| SWATdoctR | Provides run_swat_verification() and the diagnostic plotting functions used by workflow.R. |
| grid | Used directly for grid.newpage(), viewport(), pushViewport(), and popViewport() when arranging plots in the PDF device. |
| gridExtra | Loaded by workflow.R. The shown script does not explicitly call gridExtra functions, but it may support grid-based plot handling or earlier variants of the workflow. |
| SWAT+ executable and valid clean_setup files | run_swat_verification() must be able to run the SWAT+ model from the clean_setup folder. |

# Step-by-step explanation of workflow.R

## Step 1 — Load packages

library(SWATdoctR)

library(grid)

library(gridExtra)

The script loads SWATdoctR for verification runs and plots. It also loads grid and gridExtra for multi-page PDF plotting. grid functions are used explicitly when creating a fresh page, defining a viewport, printing a plot into that viewport, and closing the viewport.

## Step 2 — Define three verification simulations

The script performs three separate runs of the same clean_setup model. Each call uses the same project path, output groups, date range, warm-up skip, and folder-cleaning behavior. The only intended difference is the nostress setting.

| Run object | Script setting | Project path | Outputs read | Date range | Warm-up / skip | Folder behavior |
| --- | --- | --- | --- | --- | --- | --- |
| Run_1 | nostress = 0 | ./clean_setup | wb, mgt, plt | 2004-01-01 to 2024-12-31 | years_skip = 3 | keep_folder = FALSE |
| Run_2 | nostress = 1 | ./clean_setup | wb, mgt, plt | 2004-01-01 to 2024-12-31 | years_skip = 3 | keep_folder = FALSE |
| Run_3 | nostress = 2 | ./clean_setup | wb, mgt, plt | 2004-01-01 to 2024-12-31 | years_skip = 3 | keep_folder = FALSE |

Run_1 <- run_swat_verification(

"./clean_setup",

outputs = c("wb", "mgt", "plt"),

start_date = 20040101,

end_date = 20231231,

years_skip = 3,

nostress = 0,

keep_folder = FALSE

)

The start and end dates are consistent with the inspected clean_setup/time.sim file, which is configured with yrc_start = 2004 and yrc_end = 2024. The first three years are skipped from written outputs, so diagnostics emphasize the post-warm-up period.

## Step 3 — Combine the three run objects

sim_list <- list(all_str = Run_1, no_str = Run_2, nutrient_str = Run_3)

The combined list allows SWATdoctR plotting functions to compare the same variable across different stress settings. This is used later for yield and PHU comparison plots. For teaching, the label nutrient_str should be reviewed because it may not clearly communicate what Run_3 represents in the current SWATdoctR/SWAT+ version.

## Step 4 — Open a multi-page PDF device

pdf("plots.pdf", width = 15, height = 25)

All subsequent plots are written to a single PDF file named plots.pdf. The dimensions are large: 15 inches wide and 25 inches high. This is suitable for complex plots, but the resulting PDF pages may be large. For a workshop, it may be useful to create smaller separate files or save individual PNG files as well.

## Step 5 — Plot annual climate diagnostics

grid.newpage()

pushViewport(viewport(width = 0.8, height = 1))

print(plot_climate_annual(Run_1), newpage = FALSE)

popViewport()

This plot checks the annual climate variables from Run_1. It is intended to reveal whether climate inputs were read and interpreted plausibly. Examples of problems to look for are missing years, unrealistic precipitation totals, implausible temperatures, or sudden discontinuities caused by input formatting problems.

## Step 6 — Plot monthly snow diagnostics

grid.newpage()

pushViewport(viewport(width = 0.8, height = 0.3))

print(plot_monthly_snow(Run_1), newpage = FALSE)

popViewport()

The snow diagnostic focuses on monthly snow behavior. This is useful in cold or mixed climates because unrealistic snow accumulation or melt timing can indicate problems in climate inputs, temperature thresholds, elevation effects, or snow-related model settings.

## Step 7 — Plot simplified and detailed water balance

print(plot_waterbalance(Run_1, simplified = TRUE), newpage = FALSE)

print(plot_waterbalance(Run_1), newpage = FALSE)

Two water-balance plots are generated. The simplified version is for quick screening, while the full version gives more detail. These plots should be checked for implausible magnitudes, missing components, unrealistic dominance of one term, or symptoms of disconnected routing.

## Step 8 — Plot yield and PHU at harvest/kill for Run_1

plot_variable_at_harvkill(Run_1, variable = 'yield')

plot_variable_at_harvkill(Run_1, variable = 'phu')

These plots summarize crop yield and potential heat-unit fraction at harvest/kill events. PHU values close to crop maturity are important for checking whether growing periods and crop parameters are compatible with the climate and management schedule. Very low yields, extreme yields, PHU values far below maturity, or very large PHU values should be investigated.

## Step 9 — Compare yield and PHU across stress settings

plot_variable_at_harvkill(sim_list, variable = 'yield')

plot_variable_at_harvkill(sim_list, variable = 'phu')

These plots compare Run_1, Run_2, and Run_3. Their role is to show how stress settings affect yield and crop maturity indicators. For example, a large difference between no-stress and stress-active runs can indicate that stress factors, fertilizer inputs, irrigation assumptions, crop parameters, or soil properties need further inspection.

## Step 10 — Plot stress indicators for Run_1

plot_variable_at_harvkill(Run_1, variable = 'stress')

This plot checks stress values at harvest/kill events when all stresses are active. It should help identify whether low yield is linked to water stress, aeration stress, temperature stress, nitrogen stress, or phosphorus stress, depending on the variables made available by SWATdoctR.

## Step 11 — Optional point source plot

# plot_ps(Run_1)

The point-source plot is commented out in the repository script. It should only be enabled if the model contains point-source inputs and the relevant outputs were extracted. The official SWATdoctR workflow recommends checking point sources and tile drainage if these processes are implemented.

## Step 12 — Plot daily HRU plant variables for selected HRUs

plot_hru_pw_day(

Run_1,

hru_id = c(103, 117, 160, 205, 274),

var = c('lai', 'bioms'),

years = 2010:2023,

add_crop = TRUE

)

This is a detailed diagnostic for selected HRUs. It plots daily plant-water/plant-growth variables, specifically LAI and biomass, for HRUs 103, 117, 160, 205, and 274 from 2010 to 2024. add_crop = TRUE overlays crop information, which helps link growth behavior to the active crop and management schedule.

## Step 13 — Close the PDF

dev.off()

This closes the PDF graphics device. If this line is not executed because of an error in an earlier plot, plots.pdf may be incomplete or locked by the R session. In such a case, run dev.off() manually until no PDF device remains open, then rerun the failed section.

# Expected outputs

| Output | Created by | Content / use |
| --- | --- | --- |
| Run_1 | run_swat_verification() | In-memory list of extracted SWAT+ outputs for all stress factors active, according to workflow.R comments. |
| Run_2 | run_swat_verification() | In-memory list of extracted SWAT+ outputs for no-stress verification. |
| Run_3 | run_swat_verification() | In-memory list of extracted SWAT+ outputs for the third stress configuration; verify exact semantics of nostress = 2 for the installed version. |
| sim_list | base R list() | Combined object used for cross-run yield and PHU comparisons. |
| plots.pdf | pdf() graphics device | Multi-page diagnostic PDF containing climate, snow, water balance, harvest/kill, stress, and HRU daily plots. |
| Temporary verification run folder | SWATdoctR internals | Normally deleted because keep_folder = FALSE. Set keep_folder = TRUE only when debugging run files. |

# How to interpret the generated plots

| Plot | What to check | Possible issue if abnormal |
| --- | --- | --- |
| plot_climate_annual(Run_1) | Annual precipitation, temperature, and other climate summaries. | Wrong units, missing years, incorrect station assignment, or corrupted weather input. |
| plot_monthly_snow(Run_1) | Seasonal snow accumulation and melt behavior. | Temperature/snow parameter inconsistency, implausible snowfall/melt, or elevation-related mismatch. |
| plot_waterbalance(Run_1, simplified = TRUE) | First-pass water-balance plausibility. | Disconnected hydrology, unrealistic evapotranspiration, excessive runoff, or missing outputs. |
| plot_waterbalance(Run_1) | Detailed water-balance components. | More specific diagnosis of the same water-balance problems. |
| plot_variable_at_harvkill(..., 'yield') | Yield at crop harvest/kill events. | Management schedule, crop parameters, stress limitation, or harvest operation problem. |
| plot_variable_at_harvkill(..., 'phu') | Potential heat-unit fraction at harvest/kill. | Growing season too short/long, incorrect PHU, incorrect planting/harvest timing, or crop mismatch. |
| plot_variable_at_harvkill(..., 'stress') | Stress factors at harvest/kill. | Water, aeration, temperature, nitrogen, or phosphorus limitation may be dominating. |
| plot_hru_pw_day() | Daily LAI and biomass for selected HRUs. | Crop development anomaly, wrong crop transition, failed planting/harvest, or unrealistic biomass accumulation. |